AtlasIED IsoFlare FS-D Series Revit family file instructions

IsoFlare FS-D Surface Mount Series Revit family file covers the following models:

- FS-4D-W

- FS-4D-B

- FS-6D-W

- FS-6D-B

There are two Revit family files for the FS-D series loudspeakers:

- IsoFlare FS-D Surface Mount Series with Pivoting Wall Bracket.rfa

- IsoFlare FS-D Surface Mount Series with Yoke Bracket.rfa

These are “assembly” families which include all models of the FS series loudspeakers in both white and black colors. The mounting brackets associated with the filenames are included.

These are “Face Based” families and can be placed on any suitable face in a project.

The .rfa family file and the .txt file must be in the same directory to work correctly.

When you first load the Family into your project, you will see a dialog box listing all the available models (Types) in the Family along with all the data for each type.

The user may select any combination of Types desired for the project. This may be just one or all, depending on the project’s needs.

The family and selected type(s) will show up in the “Communication Devices” section of your project browser’s “Families” section.

After inserting, you may orient the loudspeaker as desired per instance using the orientation parameters under “Model Properties”

The Pivoting Wall Bracket Family includes Azimuth, Pitch, and a Yes/No switch (Horizontal) to rotate the enclosure 90°. The Yoke Bracket Family includes Azimuth and Rotation parameters. Both Families include a visibility switch “Remove Logo”.

The Family also includes the industry standard REP (Required Electrical Power) calculation to calculate the minimum amplifier power needed to achieve the level needed at the listener. Enter the level needed in the “Desired Level @ Listener” parameter, The distance from the loudspeaker to the farthest listener of interest in the “D2” parameter, and desired “Peak Factor”. The Family will then report the minimum power required in the “REP” parameter.

| FS-4D – with Pivot Bracket | FS-4D – with Yoke Bracket |
| --- | --- |
| FS-6D – with Pivot Bracket | FS-6D – with Yoke Bracket |